ANNEXURE - D

Inspection and Test Plan

Supplier have to do inspection and test plan as per the following tables

List of aberrvations

| ABBREVIATIONS | H – HOLD Hr- RANDOM-HOLD W-WITNESS R- REVIEW PT-PENETRANT TEST RT- RADIOGRAPHY TEST P.O- PURCHASE ORDER IR- INSPECTION REPORT IPS- INSPECTION PROCESS SHEET NDT-NON-DESTRUCTIVE TESTING WPS-WELDING PROCEDURE SPECIFICATION PQR-PROCEDURE QUALIFICATION RECORD |
| --- | --- |

Hold Point: A point beyond which an activity must not proceed without the approval of Keltron.

Witness Point: Stage of inspection beyond which work may proceed but inspection authority should be able to inspect later and signoff.

INSPECTION AND TEST PLAN

| SL.NO. | INSPECTION ACTIVITY | CHARACTERISTICS | QUANTUM OF CHECK | REFERENCE STANDARD | ACCEPTANCE STANDARD | FORMAT OF REPORT | SCOPE OF INSPECTION |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | VENDOR | KELTRON |  |  |
| 1 | MATERIAL IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| 1.1 | TITANIUM PLATE RODS FOR ALL COMPONENTS | - VISUAL INSPECTION - DIMENSIONAL INSPECTION - IDENTIFICATION MARK VERIFICATION - TRANSFER MARK VERIFICATION - UT TO BE CONDUCTED ON RAW MATERIAL | 100% | DRAWING | DRAWING/ MATERIAL SPECIFICATION | DETAILED INSPECTION REPORT | H | R |  |  |
| 2 | WELDING PROCEDURES & WELDER QUALIFICATION | - ALL VARIABLES - REVIEW OF WPS/PQR | 100% | DRAWING/ ASME SEC IX | DRAWING/ ASME SEC IX | WPS & PQR | H | R |  |  |
| 3 | FABRICATION |  |  |  |  |  |  |  |  |  |
| 3.1 | MACHINED COMPONENT | - VISUAL AND DIMENSIONAL INSPECTION - TAPPED HOLE THREAD INSPECTION | 100% | DRAWING | DRAWING | INSPECTION REPORT | H | R |  |  |
| 3.2 | SHELL LONG SEAM FIT UP | - VISUAL AND DIMENSIONAL INSPECTION - EDGE PREPARATION - MISMATCH & STRAIGHTNESS | 100% | DRAWING/ ASME SEC VIII DIV.I | DRAWING/ ASME SEC VIII DIV.I | INSPECTION PROCESS SHEET | H | R |  |  |
| 3.3 | SHELL CIRCULAR. SEAM FIT UP (SHELL TO SHELL/DOME) | - VISUAL AND DIMENSIONAL INSPECTION - EDGE PREPARATION - MISMATCH & STRAIGHTNESS | 100% |  |  |  | H | R |  |  |
| 3.4 | FIT UP OF MACHINED COMPONENTS | - VISUAL AND DIMENSIONAL INSPECTION - EDGE PREPARATION - ORIENTATION | 100% |  |  |  | H | R |  |  |

INSPECTION AND TEST PLAN

| SL.NO. | INSPECTION ACTIVITY | CHARACTERISTICS | QUANTUM OF CHECK | REFERENCE STANDARD | ACCEPTANCE STANDARD | FORMAT OF REPORT | SCOPE OF INSPECTION |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | VENDOR | KELTRON |  |  |
| 4 | EXAMINATION & TESTING |  |  |  |  |  |  |  |  |  |
| 4.1 | INSPECTION AFTER WELDING | - WELD VISUAL INSPECTION - WELD DIMENSIONAL INSPECTION | 100% | DRAWING, P.O. SPEC | DRAWING, P.O SPEC | INSPECTION PROCESS SHEET | H | R |  |  |
| 4.2 | NDT | - PT – ON BRANCH WELDS | 100% | DRAWING, P.O SPECIFICATION | DRAWING, P.O SPECIFICATION | PT REPORT | H | R |  |  |
| 4.3 | NDT | - REVIEW OF RT FILMS ON BUTT WELDS | 100% | DRAWING, P.O SPECIFICATION | DRAWING, P.O SPECIFICATION | RT REPORT | H | R |  |  |
| 5 | FINAL INSPECTION | - REVIEW OF ALL INSPECTION DOCUMENTS AS PER INSPECTION TEST PLAN. - FINAL DIMENSIONAL INSPECTION | 100% | DRAWING, P.O SPECIFICATION | DRAWING, P.O SPECIFICATION | FINAL INSPECTION REPORT | H | H |  |  |
| 6 | FINAL CERTIFICATION | - COMPLETENESS OF EQUIPMENT - IDENTIFICATION AND VERIFICATION | 100% | DRAWING, P.O SPECIFICATION | DRAWING, P.O SPECIFICATION | INSPECTION RELEASE NOTE | H | H |  |  |
|  | DOCUMENTATION | 1) MATERIAL TRACTABILITY AND SUMMERY SHEET 2) MATERIAL TEST CERTIFICATES 3) NDT REPORTS – RT, PT REPORTS 4) FINAL DIMENSION REPORT |  |  |  |  |  |  |  |  |